AWS IOT QUICK START

Contents

1.	SETUP	3

1.	Software setup	3

2.	Hardware setup	4

2.	AWS with ESP32	4

1.	Register a device	4

2.	Run the device	9

3.	AWS with Rasberry Pi	12

1.	NodeJS	12

2.	Embedded C	14

4.	REFERENCE	16

# SETUP

## Software setup

ACCOUNT SETUP

- Ask your manager for an account because it is not free

- In case you already have an account, login

- Login AWS IoT with your account

Figure 1 When you login successfully

IDE & LIBRARY SETUP

Here are some software and library setup before you can run the basic configuration and examples. It just only a few minutes to download all of them

ESP32 software setup

- Install Arduino core for ESP32 from https://github.com/espressif/arduino-esp32

- Download and install the AWS_IOT library for ESP32 https://github.com/ExploreEmbedded/Hornbill-Examples/tree/master/arduino-esp32/AWS_IOT

Raspberry Pi software setup

- Clone the AWS IoT Embedded C SDK

$ git clone https://github.com/aws/aws-iot-device-sdk-embedded-C

- Clone some external libraries, then copy their source code inside the external_libs directory of the Embedded C SDK

$ git clone https://github.com/cpputest/cpputest

$ git clone https://github.com/ARMmbed/mbedtls

- Clone the prepared NodeJS demo (This one is like a separated SDK)

$ git clone https://github.com/nbxtruong/AWS-IoT-Demo

## Hardware setup

- An ESP32

- A Raspberry Pi 3 board

- DHT22 (optional – in case you want to understand deeper)

- Some LEDs (optional – in case you want to understand deeper)

- Jumper Wires (optional – in case you want to understand deeper)

- Some Buttons or Switches (optional – in case you want to understand deeper)

- Resistor 330 Ohm and 1k (optional – in case you want to understand deeper)

- A breadboard (optional – in case you want to understand deeper)

# AWS with ESP32

## Register a device

- From your console, find Internet of Things and choose AWS IoT

- Choose Manage -> Things

- Click on Create to create a new Thing

- Select Create a single thing

- Name your Thing then select Next (I named my Thing FirstTest)

- Create a certificate by clicking Create certificate

- Download all the certificates (the root CA should be saved in Notepad under the name aws-root-ca.pem) then activate all. Remember not to give these keys to anyone.

- Your Thing is created

- Go to Security->Policies->Create to create a policy for it

- Name your policy and add statements for it (I named mine FirstTest-Policy)

- Now go back to Certificates and attach the Policy that was defined above, you can view your Things Certificates in the Security section if there are many certificates

- In the Interact section of your Thing, please mind that

## Run the device

- Open the pubSubTest example of AWS_IOT library

- Change these with yours, the CLIENT_ID can be named what ever you want, for the TOPIC_NAME, I chose the Thing Shadow update: $aws/things/<your-thing-name>/shadow/update

- Copy the containing of certificates (root-CA, certificate, private-key) into  ws_iot_certificate.c file (don’t delete the \n\).

- Add some lines of code like the image below

- Upload the code

Result: Thing Shadow updates the counter value continuously and the device also receives the value in the subscribe message.

# AWS with Rasberry Pi

## NodeJS

This section will show you how to update data to AWS Thing Shadow using Node JS. A repository has been created, on your Raspberry Pi terminal

$ git clone https://github.com/nbxtruong/AWS-IoT-Demo

$ cd AWS-IoT-Demo

$ npm install

Then create a folder call “cert” and copy all your certificates in it (root-CA, certificate, private-key)

Open ThingShadow.js file and do the following steps

- edit the all certificate names in these lines

var thingShadows = awsIot.thingShadow({

keyPath: './cert/<your-private-key>.<key-format>',

caPath: './cert/<your-ca>.<ca-key-format>',

clientId: ‘<your-client-ID>',

host: ‘<your-host-URL>'

});

- edit your device name in thingShadow.register and thingShadow.update function

$ node ThingShadow.js

Result: Thing Shadow updates the counter value continuously…now get to the Shadow section on AWS and enter the new value for welcome field and ID field

- check your result on your Raspberry terminal, the 2 yellow circles prove that the values of welcome and ID field are also updated by the device

## Embedded C

## This section will show you how to publish/subsribe and update data to Thing Shadow using the Embedded C SDK

$ cd aws-iot-device-sdk-embedded-C

## Copy your certificate, private key, and root CA certificate into the certs directory

- Go to sample_apps/subscribe_publish_sample directory and config the aws_iot_config.h as follow

- Replace the subscribe_publish_sample.c file with the subscribe_publish_sample.c attached with this document

$ cd aws-iot-device-sdk-embedded-C

$ ./subscribe_publish_sample

- Go to the Test section of your AWS Thing online, subscribe to topic sdkTest/pub and…here is what you have

Result: your message has been uploaded successfully. If you press Publish to topic, go to your Raspberry terminal

Result: your message is received by the Raspberry Pi

- In case you want to publish/subscribe to Thing Shadow, go to the shadow_sample directory, do the same configuration and run the sample

- Check your Thing Shadow and Raspberry Terminal

# REFERENCE

For more details, please have a look at

- https://docs.aws.amazon.com/iot/latest/developerguide/iot-embedded-c-sdk.html

- http://exploreembedded.com/wiki/AWS_IOT_with_Arduino_ESP32

- https://aws.amazon.com/documentation/iot/

Last but not least: please read the official document about AWS IoT Developer Guide for an extremely detailed guidance